# OBJECTIVES:

- To write a program to perform calculations.

# MATERIAL:

- Atmel Studio

# WEB SITES:

- www.microchip.com 		for Atmel Studio Software

# ACTIVITY 1

Write a program that calculates (PORTC + 4) * PORTD and sends out the result through PORTB. Consider all the values are unsigned.

# ACTIVITY 2

Write a program to calculate the result of (PORTB + PORTD)/2 and send out the result through PORTB. Consider all the values are unsigned. (Note: To divide by two, you can use shift right operation)

# ACTIVITY 3

- Find the value in R0 and R1 after the following code. What are the values kept in R0 and R1?

LDI 	R16, 10 ; Load 10 into R16

LDI 	R17, 20 ; Load 20 into R17

LDI 	R18, 30 ; Load 30 into R18

MUL	R16, R17 ; Multiply R16 * R17 (10 * 20 = 200)

ADD	R0, R18 ; Add R18 to the result in R0 (200 + 30)

Final Values: R0 = 230 R1 = 0

- Find the value in R0 and R1 after the following code.What are the values kept in R0 and R1?

LDI 	R19, 19 ; Load 19 into R19

SUBI	R19, 10 ; Subtract 10 from R19 (19 - 10 = 9)

LDI	R30, 30 ; Load 30 into R30

MUL	R30, R1 ; Multiply R30 * R19 (30 * 9 = 270)

Final Values: R0 = 14 R1 = 1

AVR hardware automatically splits multiplication results into R1 (High Byte) and R0 (Low Byte) to accommodate values larger than 255. For your $30 \times 9 = 270$ example, 270 is too big for one register, so R1 stores 1 (worth 256) and R0 stores 14, totaling 270

# ACTIVITY 4

Write a program to add 10 bytes of data and store the result in registers R30 and R31. The bytes are stored in the Program memory starting at $200. The data would look as follows:

MYDATA:	.DB	92,34,84,129,...		;pick your own data.

Note you must first bring the data from Program memory into the registers, then add them together. Use a simulator and single-step to examine the data.

# ACTIVITY 5

Write a program to add 10 bytes of Binary-Coded Decimal (BCD) data and store the result in R30 and R31. The bytes are stored in Program memory starting at $300. The data would look as follows:

MYDATA:	.DB	$92,$34,$84,$29,...	;pick your own data.

Note you must first bring the data from Program memory into the registers, then add them together. Use a simulator and single-step to examine the data.

# ACTIVITY 6

Write a program to add two BCD numbers and store the result in RAM location $100 - $104. The two multibyte items are stored in the program memory starting at $120 as following data.

.ORG  $120

DATA_1:		.DB	$54,$76,$65,$98	;number 0x98657654

DATA_2:		.DB	$93,$56,$77,$38	;number 0x38775693