# OBJECTIVES:

- To interface seven-segment to the AVR simulator.

# MATERIAL:

- Atmel Studio or the assembler of your choice.

- https://lcgamboa.github.io/js/picsimlab.html?../picsimlab_examples/ (Simulator)

# WEB SITES:

- www.microchip.com 		for Atmel Studio Software

# ACTIVITY 1

- Connect a common cathode 7-segment directly to PORTD.

- Write the following program in the AVR Studio, build and download to the picsimlab.

LDI	R20, 0xFF

OUT	DDRD, R20

LDI	R20, 0b00000111

OUT	PORTD, R20

HERE: RJMP	HERE

# ACTIVITY 2

Modify the previous program to display 9 on the 7-segment.

# ACTIVITY 3

Use a look-up table to write a subroutine that displays the value stored in R21 on the 7-segment.

# ACTIVITY 4

- Connect two 7-segments to the same port of AVR, as shown below.

- Display 23 by scanning on those two 7-segments.